USB-C Issues

Let me hop back in here for a minute. There is a big issue going around with gaming laptops right now. Many of them overly throttle their CPU performance when running on USB power. To test this we plugged in a 140 Watt USB C   charger from HP. When not using the dedicated GPU, it should allow the CPU to get up to around 100 watts leaving 30 watts for the rest of the system. Unfortunately, the CPU only drew 50 watts.  Now, when we ran a game, which uses the dedicated GPU we did see an improvement. Around 105 watts was drawn between the CPU and GPU. However, when we measured power draw from the wall. It never pulled more than 84 watts. It pulled the delta from the laptop’s battery. Meaning when you are doing something intensive while plugged into a USB charger, the battery is drained. We re-checked this for CPU only tasks, same issue. We spoke to Dell who claim these are only rated for 130 watt USB charging and that they only verified that using their own charger. So, we tried that one. The good news is that there was no battery drain, and the CPU did perform a little better. Much higher burst power and a slightly higher average watts. However, its power draw is still less than we should get. When we tested the dGPU with a game, everything fell apart. The results you are seeing on screen are real. Only 19 watts fed to the dedicated GPU resulting in 11 FPS in Forza. We tested multiple times and the 18inch also exhibited this issue. Clearly, many laptop manufacturers have work to do here. Dell has some serious work to do. These laptops have Thunderbolt 5. They should support 240 watt USB C charging. It should also correctly allocate the power. Enough on this topic back to you Cierra.